TDOT Utility Permit Requirements

The TDOT Utility Department requires the City of Lebanon to apply for a permit on your behalf. In order for the City to process your request, we will need the following items as required by TDOT and the City of Lebanon to be submitted to our offices.  Once everything is in hand, we will apply for the permit on your behalf.

TDOT SUBMITTAL REQUIREMENTS

- Plan Sets

- Three (3) sets of plans/sketches are required for all applications except roadway lighting, structure, consultant/contractor involvement and bridge encroachments where five (5) copies are required. Plans/sketches need not be large version or contain reference to items outside of TDOT’s responsibilities/Rights-of-way.

- Plans/sketches should be submitted on 11”x17” paper.

- Plans/sketches must be color coded as follows:

  - Red – Proposed facilities

  - Green – Existing facilities to be removed and/or retired in place. (Grout 2” diameter or greater.)

  - Yellow – Existing facilities to remain

  - Blue – Temporary facilities

- Plans must contain the following information:

  - Location/s within roadway Right-of-Way referenced numerically to outside driving edge of roadway (white line) and right-of-way

  - Buried facilities must show amount of cover with the use of profile and cross sections (minimum 36” cover required)

  - Location of project (Street address, log mile if available, Longitude and Latitude must be shown in decimal of degree.   Google “Earth” is especially helpful with this and can be set under tools-options and then select “decimal of degrees.

  - Show State Highway numbers, Federal Highway numbers, county and if      applicable, town or city.

  - Show the beginning and end of project referenced to the nearest tenth of log mile.

  - Note: Pencil and Paper drawings are allowed provided they legibly show all necessary information using the standard Color codes listed above.

- New appurtenances must be located as close to the Right of Way line as possible and referenced above/below the adjoining traveled way. Below ground structures must be                 structurally compliant to support machinery/vehicle travel across their axis without collapse or snag hazard.  With limited Fiber Optic exception (Fees apply) No Utility installations in areas where Controlled Access conditions exist.

- All pipeline encroachments must include an information sheet.

- All Utilities must be located as near as practical to the Right-of-way line

- Pipeline Encroachment – Form 2011

  - Link to form is available on our website

- Environmental Form

  - Link to form is available on our website.

  - Applicant to fill out everything highlighted in yellow.  Remove highlighting after execution (before sending to the City).

  - City will sign as representative.

- Utility Use & Occupancy Agreement - Form 2020

  - Link to form is available on our website.

  - Applicant to fill out everything highlighted in yellow.  Remove highlighting after execution (before sending to the City).

  - Do not fill out anything in A) highlighted in red or change the date in B).  Remove the red highlighting before execution.

- TDOT Utility Letter

  - Link to letter format is available on our website.

  - Applicant to update everything highlighted in yellow to fit the proposed project.  Remove highlighting after execution and submit in Word format to the City.

CITY OF LEBANON REQUIREMENTS

- Utility Use and Occupancy Agreement

  - This is an agreement between the City and the owner/developer.  For a commercial/industrial project, use the document titled 00Comm-Ind.  For a residential development, use the document titled 00Subd.-PUD.

- Letter of Credit

  - A Letter of Credit (LofC) in the amount of $50,000 is required.

  - The LofC should meet the City of Lebanon’s requirements (link available on our website).

  - Once work covered under the TDOT Utility Permit is complete and we have received notice from TDOT that they have conducted their final inspection, we will reduce the LofC to $5,000 and hold for a period of one year.  This will constitute a one year warranty period.